MEDICAL EQUIPMENT FOR WOMEN AND CHILD CARE

- Gynecology surgery instruments are used to treat the female reproductive system, pregnancy, and childbirth. Some common gynecology surgery instruments include:

- Vaginal speculum: Stretches the vaginal and uterine space to allow for treatment

- Tenaculum forceps: A finger ring instrument used to grasp and move the uterus or cervix during procedures like hysterectomy

- Forceps: Used to grasp the cervix or uterus during procedures like a colposcopy or hysterectomy

- Ovum forceps: Used to grasp the uterus or cervix and remove placental fragments

- Scissors: Used for delicate dissection and separation of adhesions

- Obstetrical forceps: Used to pull on, turn, or understand the fetal head

- Uterine dilators: Used during gynecology procedures

The list of Gynaecology surgery instruments is given at Annex-A.

The major aspects covered in the Indian standards for gynaecology surgery instruments include material, dimensions, finishing requirements and corrosion testing.

- MTP suction apparatus is an important device used in connection with implementation of various family welfare programmes. It is used to remove pregnancy tissue from the uterus during an early abortion or miscarriage. It consists of a suction pump, a flexible tube, and a collection chamber. The procedure is called vacuum aspiration.

The safety requirements in IS 7080 series for MTP apparatus includes the following:

| Sr No. | Requirements |
| --- | --- |
| 1 | Protection against access to live parts |
| 2 | Leakage current and Electrical strength at operating temperature |
| 3 | Stability and mechanical hazards |
| 4 | Mechanical strength |
| 5 | Supply connection and external flexible cords |
| 6 | Terminals |
| 7 | Earthing |
| 8 | Screws and connections |
| 9 | Resistance to rusting |

- Medical equipment for infant care

The health and safety of women and children are covered by all medical equipment available today. There are specific medical equipment specially intended for neonatal and infant care. The following are discussed in brief:

- An infant incubator is a device that provides a safe, controlled environment for an infant to grow and develop. Incubators are often used in neonatal intensive care units (NICU).

- Infant warmers use a heated mattress and overhead heat lamp to provide warmth from underneath the infant. Newborns, especially preterm infants, are at risk of losing body heat quickly because of their small size and lack of body fat. Infant warmers provide easy access to newborns who need critical medical care. Incubators are used for infants who need prolonged thermal support. Infant warmers are used when ease of access is critical.

- Infant phototherapy equipment is a medical device that uses a radiation spectrum in the range between 400 nm and 550 nm for reducing the concentration of bilirubin in the body of infants (i.e. to treat jaundice).

The IS 13450 series of standards are a set of standards that establish safety and performance requirements for medical electrical equipment. The standards in this series for particular requirements for a type of medical equipment refer to IS 13450 (Part 1) for general safety.

For medical equipment used in infant care, the following Indian Standards apply:

IS 13450 (Part 2/Sec 19) : 2023 Medical Electrical Equipment Part 2 Particular Requirements for Basic Safety and Essential Performance Section 19 Infant Incubators

IS 13450 (Part 2/Sec 21) : 2023 Medical Electrical Equipment Part 2 Particular Requirements for Basic Safety and Essential Performance Section 21 Infant Radiant Warmers

IS 13450 (Part 2/Sec 50) : 2022 Medical Electrical Equipment Part 2 Particular Requirements for Basic Safety and Essential Performance Section 50 Infant phototherapy equipment

The major requirements and safety aspects covered by above standards are:

- Construction

- Protection against electrical and mechanical hazards

- Protection against unwanted and excessive radiation hazards

- Protection against excessive temperatures and other hazards

- Accuracy of controls and instruments and protection against hazardous outputs

ANNEX-A

| IS No. | Title |
| --- | --- |
| IS 5829: 1982 | Specification for sound uterine |
| IS 5849: 1970 | Retractor Anterior Vaginal Wall Sim’s pattern |
| IS 5906: 1970 | Speculum Vaginal Cusco’s pattern |
| IS 6111: 1971 | Specification for curette uterine Suction Type |
| IS 6112: 1971 | Speculum Vaginal Double Ended Sim’s Pattern |
| IS 6114: 1991 | Gynaecological instruments forceps uterine vulsellum - Shape and dimensions |
| IS 6115: 1991 | Gynaecological instruments – Forceps uterine tenaculum – Shape and dimensions |
| IS 6373: 1971 | Specification for sucker mucus |
| IS 6499: 1971 | Curette Uterine Double-ended Sharp Sim’s Pattern |
| IS 6505: 1971 | Curette Uterine Double-ended Sharp and Blunt Sim’s Pattern |
| IS 6510: 1971 | Specification for curette endometrial biopsy suction type |
| IS 6565: 1972 | Stethoscope foetoscope for Foetal Heart Sounds Pinard’s Pattern |
| IS 6578: 1992 | Obstetrics instruments – Forceps ovum – Shape and dimensions |
| IS 6584: 1972 | Dilators Uterine Single Ended Hegar’s Pattern |
| IS 6589: 1992 | Gynaecological Instruments - Forceps uterine dressing Bozemann s pattern - Shape and dimensions |
| IS 6957: 1973 | Specification for curette blunt uterine |
| IS 6959: 1973 | Specification for scissors embryotomy curved on flat |
| IS 6960: 1973 | Specification for catheters metal female |
| IS 6999: 1973 | Specification for Cannula Artificial Insemination |
| IS 7080 (Part 1): 1992 | MTP suction apparatus – Specification Part 1 manually operated Second Revision |
| IS 7080 (Part 2): 1992 | MTP suction apparatus – Specification Part 2 electrical and manually operated First Revision |
| IS 7080 (Part 3): 1992 | MTP suction apparatus specification Part 3 electrically operated |
| IS 7100: 1973 | Hook Decapitation Jardine’s Pattern |
| IS 7103: 1973 | Specification for scissors episiotomy |
| IS 7115: 1973 | Specification for apparatus tubal patency |
| IS 7116: 1973 | Specification for forceps midwifery short |
| IS 7117: 1973 | Specification for scissors umbilical cord |
| IS 7432: 1974 | Clamp Myomectomy Bonney’s Pattern |
| IS 7433: 1992 | Obstetrics Instruments – Forceps round ligament Bonney’s pattern – Shape and dimensions |
| IS 7475: 1992 | Obstetric instruments – Clamp vaginal angled – Shape and dimensions (First Revision) |
| IS 7480: 1974 | Perforator Simpson’s Pattern |
| IS 7484: 1974 | Retractor Vaginal Purandare’s Pattern |
| IS 7725: 1975 | Hook IUCD Shirodkar’s Pattern |
| IS 7735: 1992 | Forceps sponge holding – Shape and dimensions (First Revision) |
| IS 7736: 1975 | Specification for screw myoma |
| IS 7737: 1975 | Specification for forceps punch cervical biopsy |
| IS 7964: 1992 | Obstetric instruments – Forceps caesarean section haemostasis green – Armytage pattern – Shape and dimensions (First Revision) |
| IS 7981 (Part 1): 1976 | Instruments Tuboplasty – Part I Occluder Cervical Shirodkar’s Pattern |
| IS 7981 (Part 2): 1976 | Specification For Instruments Tuboplasty – Part II Cannula Shirodkar’s Pattern |
| IS 7981 (Part 3): 1976 | Instruments Tuboplasty – Part III Guide Shirodkar’s Pattern |
| IS 7981 (Part 4): 1976 | Instruments Tuboplasty – Part IV Probe Single Ended Shirodkar’s Pattern |
| IS 7981 (Part 5): 1976 | Instruments Tuboplasty – Part V Probe Double-Ended Shirodkar’s Pattern |
| IS 8313: 1977 | Specification for cannula flexible karman type |
| IS 8459: 1977 | Specification for menstrual regulation syringe |
| IS 9756: 1981 | Specification for cups vacuum extraction |
| IS 10156: 1982 | Specification for cannula intra-uterine hysterosalpingography |
| IS 10545: 1992 | Gynaecological and obstetric instruments - Clamp curved on flat Heaney s pattern - Shape and dimensions |
| IS 12256: 1988 | Specification for curette uterine biopsy angled |
| IS 12270: 1988 | Cannula Spackmann’s Pattern |
| IS 12271: 1988 | Specification for laproscopic trocar and cannula |
| IS 18562 : 2024 | PPIUCD Insertion Forceps |