ASSISITIVE PRODUCTS

Assistive products are devices, equipment, software, or instruments that help people with disabilities perform tasks, communicate, and participate in daily life.

Examples of assistive products

- Mobility aids: Wheelchairs, canes, crutches, walkers, scooters, prosthetics, and orthotic devices

- Vision aids: Eyeglasses, magnifiers, screen readers, braille displays, and wearable devices

- Hearing aids: Devices that amplify sounds to help people with hearing impairments communicate

- Communication aids: Speech-to-text software, talking calculators, communication boards, and sign language interpreters

- Cognitive aids: Pill organizers, whiteboards, and educational software

- Self-care aids: Toilet and shower chairs, absorbent cloths, and power lifts

Assistive products can help people with a variety of disabilities, including low vision, hearing loss, and mobility challenges.

Mobility Aids: Wheelchairs, canes, crutches, walkers, scooters, prosthetics, and orthotic devices

- Wheelchairs: The type classification of the wheelchairs according to the means of propulsion/steering are as follows:

  - Type 1 Attendant controlled–non-powered

  - Type 2 Non-powered direct drive on rear wheels, bimanual

  - Type 3 Non-powered direct drive on front wheels, bimanual

  - Type 4 Non-powered lever drive, bimanual

  - Type 5 Non-powered single-sided drive

  - Type 6 Non-powered foot propulsion

  - Type 7 Attendant controlled — powered

  - Type 8 Electromotor for drive, manual steering

  - Type 9 Electromotor for drive, power steering

  - Type 0 Others

Important aspects of safety and performance for Wheelchairs:

- Material Requirements for: Frame, Seat and Backrest, Footrests, Rear Wheels

- Construction

- Finishing requirements

- Tests:

- Hazard Running Test- The effect of this test is to subject the framework of the wheelchair to simulated conditions similar to the worst conditions ever likely to be met in use.

- Load Test

- Stability Test- This test is to check the stability of wheelchair so that the wheelchair resists toppling.

- Attachments and Accessories like  Heel loops, Toe straps, Calf pads, Crutch holder, Pegs on hand rims, etc. Various accessories are required to cater to the needs of different categories of patients. All the attachments incorporating such accessories shall be provided with the basic model

The relevant Indian Standards for Wheelchairs and Tricycles are listed below:

| IS No | Title |
| --- | --- |
| IS 7454 : 2024 | Rehabilitation Equipment - Wheelchairs, Folding, Adult Size |
| IS 8086 : 2024 | Rehabilitation Equipment - Wheelchairs, Folding, Junior Size |
| IS 8088 : 2019 | Tricycle, hand propelled |
| IS 17154 : 2024 | Battery Operated Motorized Tricycle |
| IS 17155 : 2019 | Tricycle Single Hand Propelled Right Left Junior Size |

- Adjustable Axillary Crutches:

Adjustable axillary crutches are underarm crutches that can be adjusted to help people with mobility challenges. Generally, they are used to support a person's weight when one leg is not bearing weight. They transfer weight from the legs to the upper body.

Important aspects of safety and performance for Crutches:

- Material requirements- Aluminum/Wood, mild steel (for metallic components), Leather cloth (for arm piece), Foam rubber (for padding of arm piece), integral PU foam with engineering plastic insert (alternative for arm piece and hand piece)

- Dimensions

- Workmanship and Finishing requirements- Smooth surfaces, edges even and rounded, painting/protective coating etc.

- Tests:

- Strength Test

- Drop Test

- Walkers:

  - Have four legs that touch the ground

  - Require the user to lift the walker with each step

  - Provide support for balance and stability

  - Are often used by people who have difficulty bearing weight on one or both legs

  - Are generally less expensive than rollators

- Rollators

  - Have wheels on all legs, allowing the user to push the device instead of lifting it

  - Provide support without slowing down the user's walking pace

  - Are often used by people who need help with balance and stability

  - Can be equipped with a seat, allowing the user to rest

  - Can be more difficult to maneuver in tight spaces

Important aspects of safety and performance for Rollators:

- General Requirements:

      - Structure requirements- A rollator shall be designed to be manoeuvrable for indoor or outdoor use or a combination of the two.

      - User mass/load limit- The maximum user mass shall be specified by the manufacturer. For load carrying accessories, the load capacity of the accessories shall be specified by the manufacturer.

      - Brake effectiveness- Rollators shall have running brakes that are easy to operate by the user when the rollator is in motion. Brake performance shall not be adversely affected by folding, unfolding or adjusting actions.

      - Durability of brakes

  - Materials:

    - Flammability- Risk of flammability that can affect user safety shall be assessed by the manufacturer in the risk analysis.

    - Biocompatibility and toxicity- Materials that come into contact with the human body shall be assessed for biocompatibility using the guidance in ISO 10993-1.

    - Resistance to corrosion- Assistive products for walking that are identified to be at risk of corrosion shall be sufficiently protected against corrosion (Tested using salt spray test).

  - Ingress of liquids:

If liquid can come unintentionally into any cavities or enclosure, it shall be able to drain through drain holes again. The hazards that can be caused by the ingress of liquids shall be assessed in the risk analysis

  - Temperatures of parts that come in contact with human skin:

The risk analysis shall identify hazards and evaluate the risks associated with the surface temperature of parts that can come into contact with human skin during the intended conditions of use.

      - Safety of moving parts

      - Folding, adjusting and locking mechanisms

      - Static stability

The relevant Indian Standards for aids for walking are listed below:

| IS No | Title |
| --- | --- |
| IS 18653 (Part 1) : 2024 | Assistive products for walking manipulated by both arms Requirements and test methods Part 1: Walking frames |
| IS 18653 (Part 2) : 2024 | Assistive products for walking manipulated by both arms - Requirements and test methods Part 2 Rollators |
| IS 18653 (Part 3) : 2024 | Walking aids manipulated by both arms Requirements and test methods Part 3: Walking tables |
| IS 5143 : 2024 | Adjustable Axillary Crutches |

Orthosis

- A brace or splint that supports a body part

- Used to stabilize, support, or remind a body part to move

- Can be prefabricated or custom-made

- Used for many musculo-skeletal problems

- Examples include wrist/hand orthoses, hip orthoses, and cranial remolding helmets

Prosthesis

- An artificial limb that replaces a missing body part

- Used to restore the function and appearance of an amputated limb

- Can be designed to replace a specific body part, such as an upper limb or lower limb

- Used for traumatic injuries, birth defects, and medical conditions

External prosthesis is defined as an externally applied device consisting of a single component or an assembly of components used to replace wholly, or in part, an absent or deficient lower or upper-limb segment.

External orthosis is defined as an externally applied device consisting of a single component or an assembly of components applied to the whole or part of the lower limb, upper-limb, trunk, head or neck and their intermediate joints to assist the neuro-muscular and skeletal systems

The above definitions are as per IS/ISO 22523 : 2006 External Limb Prostheses and External Orthoses — Requirements and Test Methods. This standard specifies the following requirements for external limb prostheses and orthoses:

- Requirements for materials: Flammability of materials and toxicity of combustion products, Biocompatibility, contaminants and residues, Infection and microbiological contamination, Resistance to corrosion and degradation

- Electrical safety- for battery-powered devices

- Surface temperature

- Sterility

- Design requirements: Safety of moving parts, safety of connections

- Mechanical requirements

Jaipur foot is a product that widely used as part of prosthetic system. Some of these artificial limbs are also provided under Assistance to Disabled persons for purchasing/fitting of aids/appliances (ADIP) scheme of Ministry of Social Justice and Empowerment. The Jaipur foot prosthesis provides movement in three planes, namely:

  - Dorsiflexion and plantar flexion;

  - Pronation and supination; and

  - Transverse rotation at the ankle.

The prosthesis is hand-fabricated, using locally available raw materials of variable quality and fitted on the amputee. In practice, the wide safety margins inherent in the design and fabricated methodology of the prosthesis adequately compensate for all but gross variations in material or construction. For conformity that the Jaipur foot has flexibility in several planes, the detailed tests are incorporated in the standard IS 17034 : 2018.

﻿The complete list of standards published by BIS in relation to Assistive Products are given below:

| S.No. | IS No. | Title |
| --- | --- | --- |
| Wheelchairs and Tricycles: |  |  |
| 1 | IS 7454 : 2024 | Wheelchairs, Folding, Adult Size |
| 2 | IS 8086 : 2024 | Wheelchairs, Folding, Junior Size |
| 3 | IS 8088 : 2019 | Tricycle, hand propelled |
| 5 | IS 17154 : 2024 | Battery Operated Motorized Tricycle |
| 6 | IS 17155 : 2019 | Tricycle, Single, Hand-Propelled, Right/Left, Junior Size |
| Aids for visually impaired: |  |  |
| 7 | IS 11279 : 2024 | Braille Slate — Specification |
| 8 | IS 11646 (Part 1) : 2003 | Cane for visually handicapped Part 1 rigid |
| 9 | IS 11646 (Part 2) : 1986 | Cane for visually handicapped: Part 2 folding type |
| 10 | IS 11647 : 1986 | Braille paper |
| 11 | IS 12152 : 1987 | Pocket frame, braille writing |
| 12 | IS 12184 : 1987 | Stylus for braille writing |
| 13 | IS 12439 : 1988 | Signature guide for visually impaired |
| 14 | IS 13822 : 1993 | Braille duplicating sheet |
| 15 | IS 13837 : 1993 | Braille duplicating machine |
| 16 | IS 14429 : 1997 | Braille shorthand machine |
| Aids for walking: |  |  |
| 17 | IS 13017 : 1991 | Walker rollator |
| 18 | IS 18653 (Part 1) : 2024/ISO 11199-1: 2021 | Assistive products for walking manipulated by both arms Requirements and test methods Part 1: Walking frames |
| 19 | IS 18653 (Part 2) : 2024/ISO 11199-2:2021 | Assistive products for walking manipulated by both arms - Requirements and test methods Part 2 Rollators |
| 20 | IS 18653 (Part 3) : 2024/ISO 11199-3:2005 | Walking aids manipulated by both arms Requirements and test methods Part 3: Walking tables |
| 21 | IS 5143 : 2024 | Adjustable Axillary Crutches |
| Artificial limbs (Prosthesis and Orthosis): |  |  |
| 22 | IS 17034 : 2018 | Jaipur foot |
| 23 | IS 12664 (Part 1) : 2003 | Artificial limbs - Sach foot for lower extremity prostheses: Part 1 design and dimensions (First Revision) |
| 24 | IS/ISO 22523 : 2006 | External limb prostheses and external orthoses - Requirements and test methods |
| Aids for incontinence: |  |  |
| 25 | IS 15376 (Part 1) : 2003/ISO 8670-1 | Ostomy collection bags: Part 1 vocabulary |
| 26 | IS 15376 (Part 2) : 2003/ISO 8670-2 | Ostomy collection bags: Part 2 requirements and test methods |
| 27 | IS/ISO 16021 : 2000 | Urine-Absorbing Aids — Basic Principles for Evaluation of Single-Use Adult-Incontinence-Absorbing Aids from the Perspective of Users and Caregivers |
| 28 | IS/ISO 8669-2 : 1996 | Urine Collection Bags Part 2 Requirements and Test Methods |
| Other important standards pertaining to assistive technology: |  |  |
| 29 | IS 18660 : 2024/ISO 19029:2016 | Accessible design Auditory guiding signals in public facilities |
| 30 | IS 18831 : 2024/ISO 17966:2016 | Assistive products for personal hygiene that support users - Requirements and Methods of Test |
| 31 | IS/ISO 21856 : 2022 | Assistive products General requirements and test methods |